System Information

Machine: Hot Plate

Manufacturer: Systems Integration

Model #:  7200-1453-rev.1

Operating and Maintenance Procedures

1.	Plug in hotplate.  This automatically turns it on.

2.	Set the desired temperature with the controller on the side of the hotplate.

3.	When at desired temperature, set wafer on three prongs on plate.

4.	Press the DOWN button.

5.	Time your bake (there is not a built in timer)

6.	When time is up, press the UP button

7.	Unplug hotplate